NORMAL CHECKLIST N41459

| PREFLIGHT - PAVE Checklist PASSENGER BRIEF - Seat belts - Air vents - Fire Extinguisher - Exit Doors - Traffic/ Talking - Questions - Hobbs/Tach Time Recorded - Seats Locked - Belts and Harnesses ENGINE START - Brakes Set - Circuit Breakers - Avionics Off - Autopilot Off - Carb Heat Off - Fuel Fullest Tank - Throttle Slight - Anti Collision Light On - Master Switch On - Check Volts - Fuel Pump On - Mixture Rich - Prime As Required - Mags Start - 1000 RPM or Less - Oil Pressure - Fuel Pump Off - Check Pressure - Amps/Volts - Lights as Required | BEFORE TAXI - Avionics On - Flaps Up - Heat/Vent/Defrost - ATIS / AWOS (SNS 124.85) - Altimeter Set - GPS Programmed - Transponder ALT and Squawk - Brakes Test - Flight Instruments Checked - Radio Call - 5 P’s Check (SNS GND 121.70) RUNUP - Brakes Set - Flight Controls - Flight Instruments - Mixture Rich - Primer In & Locked - Throttle 2000 RPM - Magnetos Check - Carb Heat Check - Oil Temp - Oil Pressure - AMPS & VOLTS - Throttle Check Idle - Throttle Friction | BEFORE TAKEOFF - Flaps 0°-25° - Trim for Takeoff - Fuel Proper Tank - Fuel Pump On - Carb Heat Off - Mixture Set - Pitot Heat as Required - Heading Indicator to Compass - Seats Locked - Belts and Harnesses - Doors and Windows - Transponder ALT and Squawk - Lights as required - Time Note - Brakes Released - Abort Plan Ready - Radio call (SNS TWR 119.52) - Verify runway heading TAKEOFF - Full Throttle - RPM 2350 Minimum - Oil Pressure - Rotate @60-65mph - Flaps Up | CLIMB Vx 76mph Vy 87mph Enroute 100mph - Power - Mixture - Fuel Pump Off - Landing Light Off CRUISE - 5 P’s Check - Power Set - Mixture Adjust - Instruments - Monitor Fuel Usage DESCENT & IN RANGE - 5 P’s check - Mixture Richen - Carb Heat as Required - Fuel Proper Tank - ATIS/AWOS (SNS 124.85) - Altimeter Set - Heading Indicator to Compass - Radio Call (SNS TWR 119.52) - Airport Entry & Crew Briefing | BEFORE LANDING - GUMPS - Trim for 80mph Flaps Up Short Field @73mph with Full Flaps AFTER LANDING - Flaps Up - Carb Heat Off - Fuel Pump Off - Lights as required - Pitot Heat Off - Trim for Takeoff - Transponder ALT and Squawk - Radio Call (SNS GND 121.70) SECURING - Avionics Off - Mixture Idle Cutoff - Mags Off - Master Off - Hobbs/Tach Time Recorded - Secure Yoke - Chocks - Tie Downs - Lock Aircraft |
| --- | --- | --- | --- | --- |